IoT-BASED SMART HOME
MONITORING & AUTOMATION SYSTEM

Client Overview  •  Design, Components & Capabilities

Controller: ESP32 DevKit V1  •  Low-voltage prototype  •  Wi-Fi phone control

Version 1.1  •  August 2026  •  Compact physical model (Living / Bedroom / Kitchen / Door)

Purpose — What this document is A clear, non-technical overview of what will be built, which parts are used, how the rooms work, and what you can control from your phone. No budget tables, no wiring schematics — just the design and the features you will see in the demo.

# 1. Overview — What We Will Build

A compact, low-voltage smart-home model with four spaces — living room, bedroom, kitchen and main door — controlled by a single ESP32. Sensors are installed room-wise and wired to a central breadboard; the ESP32 handles all automation locally and exposes a Wi-Fi dashboard you open on your phone (same Wi-Fi, no app needed).

| Monitoring Temperature, humidity, motion, light level, door & gas level | Automation Lights, fan & exhaust run on sensor rules with phone override | Security Door alert when armed & gas warning with buzzer |
| --- | --- | --- |

# 2. System Architecture — Simple View

One controller, room sensors, relay-switched lights/fans, and your phone over Wi-Fi. The relay separates the ESP32 (control) from the loads (power) — the ESP32 never drives a fan/light directly.

PHONE / WEB DASHBOARD — Wi-Fi │ ▼ ESP32 DevKit V1 (central brain) Sensors ─────┼───── Relay ───── Buzzer DHT11 • PIR×2 • LDR×2 • MQ-2 • Reed CH1 Living Light • CH2 Bedroom Light • CH3 Room Fan • CH4 Exhaust

All automation runs on the ESP32 itself; the phone only monitors and sends manual commands. If Wi-Fi drops, local automation still works.

# 3. Components Included

Everything needed for the prototype — sensing, control, and the house model. All loads are low-voltage 5V DC.

| Component | Qty | Purpose in Your Home |
| --- | --- | --- |
| ESP32 DevKit V1 | 1 | Main controller + Wi-Fi — runs automation & hosts dashboard |
| DHT11 module | 1 | Measures temperature & humidity (living room) |
| PIR motion sensor (HC-SR501) | 2 | Detects person/movement — living room & bedroom |
| LDR light sensor | 2 | Measures brightness — decides if dark enough to turn lights on |
| MQ-2 gas/smoke module | 1 | Kitchen air-quality threshold — triggers exhaust + buzzer |
| Magnetic reed switch | 1 | Main-door open/closed detection |
| 4-channel relay module | 1 | Switches lights & fans safely (ESP32 only sends signal) |
| 5V mini DC fan | 2 | Room ventilation + kitchen exhaust |
| 5V LED lights | 2–4 | Living & bedroom lights (and optional kitchen light) |
| Active buzzer | 1 | Beep for door-security & gas warning |
| Breadboard + jumper wires | 1 set | Central wiring — clean, labelled, reusable |
| 5V power supply + USB cable | 1 | Powers the whole prototype |
| House model (foam/cardboard) | 1 set | 4-room physical model — sensors mounted inside |

# 4. Room-Wise Design — Where Everything Goes

Living Room  —  PIR #1 + LDR #1 + DHT11  •  Light (Relay CH1) + Fan (Relay CH3)

→ Auto light (motion + dark) • Temp-based fan • Manual override from phone

Bedroom  —  PIR #2 + LDR #2  •  Light (Relay CH2)

→ Auto light (motion + dark) — same logic as living room

Kitchen  —  MQ-2  •  Exhaust (Relay CH4) + Buzzer

→ Gas threshold → exhaust + buzzer — threshold set during demo warm-up

Main Door  —  Reed switch (magnet on door, switch on frame)

→ Shows OPEN/CLOSED • When armed, opening triggers buzzer

# 4a. Model Layout — Front & Top-Down (Real Use)

Door at front enters living room. From living you go to bedroom or kitchen. Exhaust at back wall.

# 5. What It Will Do — Features You Will See

Six things your home will do — demonstrated live with the model and your phone side by side:

| Capability | What Happens | How You Control It |
| --- | --- | --- |
| 🌡️ Temperature & Humidity | Live readings on the dashboard (living room) | View only — updates automatically |
| 💡 Auto Lights | If someone moves AND the room is dark → light turns ON; turns OFF after ~30s with no motion | Auto, or tap ON/OFF on phone |
| 🌀 Smart Fan | Turns ON when warm (~30 °C), OFF when cool (~28 °C) — avoids flickering | Auto, or manual ON/OFF |
| 🚪 Door Security | Shows CLOSED/OPEN; when armed, opening the door triggers buzzer + alert | ARM / DISARM from phone |
| 🔥 Gas / Smoke Warning | If MQ-2 crosses threshold → buzzer + kitchen exhaust turn ON | Auto — threshold set during warm-up |
| 📱 Phone Dashboard | One web page shows all sensor values and buttons (no app to install) | Wi-Fi — open in any browser |

All features work together — e.g., you can leave lights in AUTO while controlling the fan manually, or arm security at night while lights stay automatic.

# 6. How Automation Works — In Plain Language

Lights:  Motion + Dark → ON  •  No motion for a while → OFF. The LDR prevents lights turning on in a bright room.

Fan:  Two limits (30 °C and 28 °C) instead of one — so the fan doesn’t rapidly click ON/OFF near the threshold.

Door:  Reed switch with magnet: door closed = quiet; door opened while ARMED = buzzer + “DOOR OPEN” on dashboard.

Kitchen:  MQ-2 is a threshold sensor (not a lab meter). We warm it 2–3 min, note its clean-air level, then set a project threshold above that.

| AUTO MODE Sensors decide — lights, fan & exhaust follow the rules above | MANUAL MODE You decide — phone buttons directly turn lights/fans ON/OFF |
| --- | --- |

# 7. Phone Dashboard — What You Will See

A single web page hosted by the ESP32 — open it from any phone/laptop on the same Wi-Fi. No separate app, no Bluetooth, no paid cloud.

SMART HOME Temperature : 28.4 °C Humidity : 63 % Motion : Living / None Door : CLOSED Gas : NORMAL ──────────────────────────────────────────────────── Living Light [ ON ] Bedroom Light [ OFF ] Room Fan [ ON ] Kitchen Exhaust [ OFF ] Mode: AUTO Security: ARMED

Top row: live temperature, humidity, motion, door and gas status.

Middle: four buttons — Living Light, Bedroom Light, Room Fan, Kitchen Exhaust.

Bottom: AUTO/MANUAL toggle and Security ARM/DISARM.

Your phone talks only to the ESP32 — not to each sensor separately. The ESP32 gathers sensor data and forwards it to the dashboard.

# 8. What You Will Receive

| Deliverable | What It Means For You |
| --- | --- |
| Physical model | 4-room house with sensors mounted and labelled, electronics underneath/behind |
| ESP32 control unit | Breadboard circuit with ESP32, dividers, reed wiring, and relay beside the board |
| Relay-controlled loads | Lights and fans switching through the relay (low-voltage 5V only) |
| Phone dashboard | Wi-Fi web page with live readings and manual controls |
| Automation | All rules above running locally on the ESP32 |
| Testing & handover | Full end-to-end demo + brief on how to operate and re-test each feature |

Optional (on request): project report, presentation slides, viva/demo preparation.

# 9. Notes

All lights and fans in the student prototype are low-voltage 5V DC — no 220V AC is introduced.

The MQ-2 is demonstrated as a relative threshold (not a calibrated gas concentration) — shown safely without flame or gas leak.

The ESP32 dashboard works on the local Wi-Fi; no internet, Bluetooth module, or paid service is needed for this prototype.

The relay module isolates power: the ESP32 sends a small control signal; the relay does the switching.

Optional future additions (camera, RFID, voice assistant, OLED, paid cloud) are possible later but are not needed to demonstrate this prototype.

ESP32  •  Monitoring  •  Automation  •  Security  •  Phone Control   —   Compact, low-cost final-year prototype